Making Life Easier in your GMC

Rapid City SD – Fall 2015

- LED lights use much less power and produce little, if any heat.

- China toilets work great

- Redesigned kitchen with a single sink and a flip up counter extension

- To keep things from sliding out of the bathroom vanity, use a closet rod on the front edge (or a clear sheetrock edge guard held on with double sided tape)

- Multiple switches to turn the water pump on/off

- Remote 12V adapter installed in bedroom for C-Pap machine

- Install a vertical Plexiglas shield next to skin to keep couch clean and dry

- Frig/Freezer remote temperature gauge allows you to know the temps at a glance

- Raise rear bed 8” and make a nice storage space below it.  Some have put gas lifts and hinges so it’s easy to get to.

- Inverter just for Microwave.  On countertop if the stove top is removed you can make a cutting board to cover the hole.

- Microwaves will fit where the oven was if you remove the back of the cabinet.  You can also mount them in the hall drawer space at eye level by removing the back of the cabinet and modifying a door.

- If you are redoing the kitchen plumbing, you can put the P-trap over by the vent stack instead of directly under the sink.  Works the same and you save a bunch of room under the sink.

- On some of the sewer lines that you might want to take apart, instead of gluing the PVC use rubber hose and hose clamps for future removal.

- Timer on hot water heater works great.

- Fantastic fan in bath is a huge improvement over the little original

- Some trucks have 24V (DC) bulbs.  Use these as night lights, they will be much dimmer

- Drop a Styrofoam cup inside a plastic cup.  Keeps ice and things cold a LONG time.

- To dry a wet bath after a shower, use a foam paint roller to roll up moisture.

- Loose rugs against the hall night lights can cause fires!!!!!  They get pretty hot.  This is believed to be the cause of at least one coach fire.

- GMC Photo site is a wealth of information for interior upgrades and repairs http://www.gmcmhphotos.com/

- A USB charger has a small power indicator LED that is enough light for a night light.  Small LED lights would work also.

- In wet bath, “DryDeck” is a great way to let the water drain out while your feet stay dry.

- Some folks have removed the dinette and replaced with table and two chairs that swivel and recline.  Storage under the chair base.

- Propane quick connectors available from a propane store so you can quickly attach a propane BBQ for outside cooking.

- A drawstring bag by the toilet is a great place to store dirty clothes.

- An extra shelf in the closet increases storage.

- A vertical piece of Plexiglas across the bottom of the closet keeps things from falling out when you open the door and you can still see what is in there.

- Cellular blinds make good replacements for curtains or mini blinds.  Some brands like Levelor have a lifetime warranty.  Many styles and colors available at big box stores.  Shades are cut to length free.

- There is some dead space behind and below the drivers side tail light area.

- Also dead space around and below the back of the fridge.

- On 77’s ad 78’s the water fill is on the drivers side rear.  Moving it to the left side allows removal of the small bulkhead and gives and extra 5” of bed length.

- If you buy something from the big box stores, make a copy of the receipt.  They quickly fade out.

- The ‘gun cabinet’ over rear window between the hanging cabinets provides some great storage.

- Replaced the dinette with a couch/bed.  Storage and drawers under it.

- Frank Jenkins makes a padded cabinet end that conceals a pistol.

- Use a shop vac to pull out all the ‘Pink Stuff” RV antifreeze.

- One person recommended using Vodka instead of Pink Stuff to winterize.

- A rod between the two rear overhead cabinets is a great place to hang clothes and wet towels.

- As a reminder that your antenna is up, write “ANTENNA” on a piece of Velcro that normally lives on the antenna crank.  When the antenna is raised, move it to the steering wheel as a reminder to lower the antenna.